Disclaimer: The official version of the law and any amendments thereto is published in Arabic in the Official Gazette. This version of the law, including amendments thereto, is provided for guidance and easy reference purposes. The Legislation & Legal Opinion Commission does not accept any liability for any discrepancy between this version and the official version as published in the Official Gazette and / or any inaccuracy or errors in the translation.

For any corrections, remarks, or suggestions, kindly contact us on translate@lloc.gov.bh

Published on the website on May 2024

Ministry of Labour

Decision No. (15) of 2014 regarding the Protection of Workers against the Hazards of Chemicals, Gases and Flammable Liquids in establishments and Worksites

The Minister of Labour:

Having reviewed the Organisation of Buildings Law promulgated by Legislative Decree No. (13) of 1977, as amended;

Legislative Decree No. (5) of 1990 regarding Civil Defence;

Legislative Decree No. (21) of 1996 regarding Environment, as amended by the Legislative Decree No. (8) of 1997;

Law No.(25) of 2009 approving the Accession of the Kingdom of Bahrain to the Convention of the International Labour Organisation No. (155) of 1981 concerning Occupational Safety and Health and Work Environment;

Labour Law for the Private Sector promulgated by Law No. (36) of 2012, particularly Article (166) thereof;

Decree No. (7) of 1983 approving the Accession to the Arab Convention No. (13) of 1981 concerning Work Environment;

And Decree No. (2) of 1994 regarding the Accession of the State of Bahrain to the Arab Agreement No. (7) of 1977 and the Recommendation No. (1) of 1977 regarding the Occupational Health and Safety;

Decision No. (31) of 1977 regarding the Identification and Organisation of services and precautionary measures required to protect workers during work from the Hazards of Highly Flammable Liquids and Liquefied Petroleum Gases;

And after consulting the Concerned Authorities,

And upon the submission of the Undersecretary of the Ministry of Labour,

Hereby Decides:

Article (1)

In applying the provisions of this Decision the following words and expressions shall have the meanings assigned to them below, unless the context requires otherwise:

• Highly Flammable Liquid: A chemical substance that ignites a fire once exposed to air or is converted, once it gets into contact with water, into a highly combusted gases.

• Cylinder: A container designed, manufactured and tested in accordance with the internationally approved compression container standards.

• Fire Resisting Structure: A store or workers room, chamber or any other closed place designed to become fire resisting in accordance with the fire resistance specifications and measures.

Article (2)

The provisions of this decision shall apply to all establishments and work sites that are subject to the provisions of the Labour Law in the Private Sector promulgated by Law No. (36) of 2012 and all existing establishments and work sites at the time of implementing this Decision shall modify their conditions in compliance with its provisions within three months from the date it shall come into effect.

Article (3)

The employer shall take the necessary precautions for the protection of his workers or those who are present at his work site from the chemical hazards and he shall provide them with the protection means from its hazards in addition to rescue means in the manner outlined in this Decision.

It shall be prohibited for the employer to encumber workers or to deduct from their wages any amount in consideration of providing such protection.

Article (4)

Every employer shall, in a suitable and effective manner, evaluate the chemical hazards at his establishment and work sites that threatens the safety and health of his workers and persons present therein or close by as follows:

1) To conduct an evaluation at least once every three years.

2) To conduct an evaluation in case of an occupational accident.

3) To conduct an evaluation in case any alteration in the establishment’s operations.

Provided the chemical hazards evaluation shall be assigned to an approved entity specialized with the type of existing chemical hazards.

Article (5)

The Occupational Health and Safety Conditions shall be observed when workers are dealing with chemicals in the following manner:

a) To isolate and separate work sites from the remaining work environment that are susceptible to chemical material hazards.

b) To replace the industrial operations using harmful substances with ones that are less harmful.

c) To conduct operations that generate harmful gases in securely sealed equipments under suitable pressure.

d) To periodically and technically maintain the measuring, technical and industrial devices to ensure their safety.

e) To provide adequate discharge and suction ventilation systems at the work site.

f) To train and educate workers with the chemical hazards at work environment.

g) To use chemicals supplied with a clear and internationally approved hazard materials identification card (MSDS).

h) To comply with the cautions and warnings issued by the chemical materials producing and manufacturing companies.

i) To conduct the periodical measurements of chemical hazards at the work environment.

j) To observe the determination of daily shift hours and a schedule may be drawn to illustrate it.

k) To provide sufficient and appropriate places for workers washing after completing the shift period in addition to supplying an emergency sprinkler.

l) To provide personal safety equipment in conformity with the nature of work.

m) To conduct the periodical check-up on workers susceptible to chemical hazards and maintain a register thereof.

Article (6)

To provide sufficient and secured safety means to prevent and escape from fire at any site where production, use or transport of any flammable chemicals, gases or liquids is taking place.

The following conditions shall be observed at a worksite to protect from fire:

a) To prevent smoking inside the work sites especially the ones containing flammable material and to post directions in this respect at each of the factory entrances in a clear and suitable indication that smoking is strictly prohibited with the exception of places where a sign indicates permission thereof.

b) To adopt the required safety measures upon carrying on welding and cutting jobs in barrels and containers that contains flammable materials or their residues.

c) To adopt necessary safety measures in the event of frictions, electromagnetic and electric charges.

d) To adopt the necessary safety and security measures upon using compressed gas cylinders provided they must be examined and supplied with safety certificates.

Article (7)

The following conditions shall be observed whenever a deposit of any exhausts liable to give rise to a risk to occur on any surface.as a result ofany process or operation involving any highly flammable liquid.

a) Steps shall be taken where reasonably practicable to prevent as far as possible the occurrence of all such residue.

b) Where any such residue occurs, effective steps shall be taken, as often as necessary to prevent danger, to remove all such residues as aforesaid and put them in a safe place.

Article (8)

The following precautions shall be observed:

a) No highly flammable liquid shall be ignited except in a place suitable for the purpose of burning that highly flammable liquid safely and by the proper use of that place or apparatus.

b) Where the sole purpose to dispose of any highly flammable liquid as waste by burning it, it shall burnt either in a place or apparatus suitable for the purpose of burning that highly flammable liquid safely and by a qualified person.

c) The prohibition indicated in Clause (a) of this Article shall not be applicable where highly flammable liquid is burnt in order to provide persons with training in fighting fire provided the burning shall be carried out by a qualified person in a safe manner and in a safe place and such training shall be carried out under the direct and continuous supervision of the qualified person.

Article (9)

The following shall be observed:

a) Where a storeroom, workroom, chamber or enclosure is fire resisting structure provision may be made in its structure for pressure relief in the case of any explosion.

b) The storeroom shall have adequate natural ventilation notwithstanding in either case, anything contained in any specification approved for the purposes of the definition of the expression (fire resisting structure).

c) To observe the conditions related to the safety of storerooms and warehouses or containers of chemicals, liquids or highly flammable gas at work sites as follows:

1) To separate the storing locations from the manufacturing or circulating locations.

2) The storeroom or chamber where the chemical materials are stored shall be a solid and non-combustible structure.

3) To classify the chemicals according to their nature, characteristics and to implement the directions that are written on the identification cards on each one and to observe their similarity while storing so as each specific type of material shall be stored independently.

4) To provide independent storage for spoiled and expired chemicals and to supply the storeroom with a laboratory in order to recycle some these chemicals and reuse them once again.

5) To provide adequate and suitable lighting.

6) To maintain a suitable and appropriate temperature to avoid igniting the flammable materials.

7) To provide suitable and secure ventilation.

8) To avoid any igniting sources in the storeroom.

9) To adopt required measures to limit fire from spreading upon occurrence in the storeroom.

10) To educate the workers with the hazards of the stored chemicals and methods of prevention thereof.

11) To observe the stores order and arrangement together with adopting measures to prevent materials from or workers falling.

12) To ensure the availability of safety supplies and personal safety equipment.

13) To draw up emergency and evacuation plan in case of leakage or fire.

d) The cabinets, boxes, warehouses and containers used at any time to store highly flammable liquids shall be closed and sealed (whether or not containing any highly flammable liquid) unless a need may arise to use, operate of maintain such cabinet, boxes, warehouses and containers.

e) To adopt all possible practical measures upon storing highly flammable liquids which may leak, spill or may be released in any method in order to avoid their leakage from their tanks or containers by containing or draining them into a suitable container, a safe place or to be treated in any other method to become safe.

Article (10)

The term (Flammable) or any other suitable term that indicates the flammable liability shall be written on every storeroom, cabinet, box or container used to store flammable liquids.

If it is impractical to place a sign on the storeroom, cabinet, box or warehouse or container, the term (highly flammable liquid) shall be clearly placed at the nearest place.

Article (11)

The following shall be observed:

a) Storage of all highly flammable gases or liquids which are not in use shall be made in accordance with the following conditions:

1) The storage shall be in suitable underground tankers, land or in the open air or warehouses or in fixed or mobile containers in safe areas.

2) It shall be in pipelines and pumps or other parts forming a totally enclosed pipeline system.

3) It shall be in suitable cylinders kept in safe areas in the open air, or where this is not reasonably practical, in a storeroom constructed of non- combustible material being a storeroom which is adequately ventilated, in a secure location or in fire resisting structure.

b) No gases or flammable liquids other than in suitable cylinders or suitable pipe-lines shall be present in any workplace and the number of gas cylinders or pipe-lines present at any one time shall be as small as is reasonably practicable having regard to the process or operations being carried thereon.

c) Gas cylinders shall be stored until such time before first required for use or manipulation as is reasonable, and when its contents have been expended shall, as reasonably practical, be removed from the workplace and stored or re-filled without delay.

The preceding provisions shall not be applicable to the following two cases:

1- The gas or liquid in the fuel tanks of vehicles or engines for the purpose of operating such vehicles or engines.

2- Any small securely closed vessel containing not more than 500 cc of flammable gas or liquid.

Article (12)

a) To observe using the least amount of a highly flammable liquid at a workplace whether in production or being used in containers, tanks, pipe-lines, equipment or appliances in the most practical way possible in view of the process or operation being carried thereon.

b) All possible practical measures must be adopted to secure that no highly flammable gases have leaked from their tanks or containers such tanks and containers must be completely sealed unless it is neccessary during their production, use, and work with or during bottling process, maintenance of the tanks or containers. Every tank or container of this type must be designed, installed and placed in a manner to practically avoid the danger of spilling or draining it immediately to another suitable container or transfer it to a safe place or process it in any danger preventative manner.

c) Any tank, warehouse or container - used to contain and flammable liquid, if not stable, and as soon as emptying it from its contents with the exception of cases where the vapours of such liquid were discharged in a safe open area or to be immediately used – should be transported without any delay to a safe place in an open area or to a store constructed in a suitable place so as such store is designed to be fire-resistant or transported to an cabinet or box in a suitable place provided such cabinet or box is made of fire resistant material.

d) All above mentioned tanks or containers which are discharged from any liquid vapour must be kept closed and maintained in a safe place, a store or cabinet or box of the aforesaid type until they are once again required to be used or transported.

Article (13)

a) No means likely to ignite vapour from any highly flammable liquid shall be present where a dangerous concentration of vapours from highly flammable liquids may reasonably be expected to be present.

b) Where in any place a dangerous concentration of vapours from highly flammable liquids may reasonably be expected to be present, any cotton waste or other material in that place has been used in such a manner as to render the cotton waste or other material liable to spontaneous combustion or is contaminated with any highly flammable liquid, shall be deposited without delay in a metal container having a suitable cover or be removed without delay to a safe place.

Article (14)

a) Neccissary steps shall be taken to prevent so far as is reasonably practical the escape of vapours from any such highly flammable liquid into the general atmosphere of the workplace or any other place.

b) Where a dangerous concentration of vapours from highly flammable liquids may reasonably be expected to be evolved in any process, the process or operation shall, where reasonably practicable, be carried on within a cabinet or other enclosure which in either case, is effective to prevent the escape of such vapours into the general atmosphere of the workplace or any other place, is adequately ventilated by mechanical means and is a fire resisting structure. In the case of a batch-loaded box-type oven used to evaporate a highly flammable liquid with a capacity of less than 1½ cubic metres and which is provided with natural ventilation to prevent dangerous concentrations of vapours, the oven is not required to be ventilated by mechanical means.

c) Where it is not reasonably practicable to comply with the requirement of the foregoing Clause where in any workroom any such dangerous concentration of vapours may reasonably be expected to be evolved in any process or operations, the workroom shall have ventilation provided by mechanical means, with ventilation adequate to remove such vapours from the room.

d) A workroom required by the foregoing Clause to have exhaust ventilation shall be a fire resisting structure with the exception of any of the following forming part thereof, that is to say, any external doors, external windows and external walls, any openings provided for ventilation and any tops or ceilings of single storey buildings or on the top floor.

e) Ventilation provided or secured in pursuance of this Article shall be kept in operation or use at all necessary times.

f) All ducts, trails and casings used in connection with the ventilation provided or secured in pursuance of this Article shall be made of fire resisting materials.

g) All exhausts of fixed ventilation devices on fixed tanks and fixed containers that contain highly flammable liquids shall be discharged to a safe place, and where necessary, shall be provided with a suitable wire-gauze effective as a flame arrestor or other suitable flame arrestor.

Article (15)

Fire-fighting appliances should be placed in visible areas and easy to be accessed.

Article (16)

The following procedures and measures shall be observed when conveying chemicals:

a) Upon using vehicles for conveying chemicals, they shall be secured and appropriate to transport materials of that nature and extent of its hazardous.

b) To place warning signs on the chemical conveyers, containers and tanks particularly the hazardous ones indicated by factories producing, importing and dealing with such materials.

c) To examine containers prior to their shipping and to carefully load and unload them.

d) Containers that are open or have contents leaking therefrom should not be transported at all.

e) To use suitable and safe methods upon transferring the contents of big containers in small containers to prevent hazardous liquids spilling.

f) Workers conveying chemicals should be experienced and knowledgeable of the nature of such materials provided having a copy of the chemical identification card to be transported.

g) Transportation workers must have the knowledge of the first aid conditions, standards and procedures.

h) To ensure that workers are wearing the suitable personal protective clothing.

i) Chemical waste containers must be conveyed from the production to the treatment place to be practically disposed of without being stored.

Article (17)

a) Upon using pipes for conveying, storing or producing highly flammable materials, the required Occupational Health and Safety Conditions must be observed particularly:

1) It should be of safe specifications and appropriate for the nature of materials used therein.

2) It shall be made of durable and non-combustible materials.

3) It shall be made, to a practical limit, from heat insulating materials.

4) Pipes must be periodically be tested and ensure their resistance to chemical and natural factors.

5) Pipes must be maintained to prevent the occurrence of any cracks or expansion that may lead to leakage therefrom.

6) Preventative methods must be used to prevent breaking of any fire upon the leakage of substance inside such pipes.

7) To educate and train workers on fighting fires.

8) To use methods, measurements and indicators showing the pressure and heat inside such pipes.

b) The following conditions shall be observed upon using or conveying gas or compressed gas cylinders:

1) Cylinders must be made from durable and resistant materials.

2) Cylinders must be filled with gas in a secure manner in order to protect workers from any hazard.

3) Quantity of gas filled shall be in conformity with the cylinder’s size to avoid explosion or leakage.

4) Workers, upon using and filling cylinders, must be trained and educated.

5) Suitable tools must be used in filling cylinders with gas.

6) To observe the cylinders validity period.

7) Continuous periodical examination and maintenance of cylinders in terms or observing the pressure indicator, its efficiency and quantity of gas therein together with maintenance of connectors attached thereto.

8) Cylinders, during welding and fabrication process, should be safe from explosion and fire hazards and must be examined to ensure that no gas is leaking therefrom.

9) A register must be maintained to indicate the cylinders maintenance date.

10) Suitable and secure storage to avoid the hazard of fire or explosion.

Article (18)

a) Where highly flammable liquids or gases are to be conveyed within a factory they shall, where it is reasonably practicable to do so, be conveyed through a totally enclosed system incorporating pipe-lines, pumps or any other similar appliances.

b) Where highly flammable liquids or gases are to be conveyed through a totally enclosed system is impractical, it shall be conveyed in containers which are designed and constructed as to avoid so far as is practical the risk of spilling.

c) A totally enclosed system used for such conveyance shall be so far as is practicable be so designed, constructed, installed, placed and maintained to avoid leakage.

Article (19)

A worker shall not commit any act or omission with the intention of preventing the execution of the instructions or the misuse or causing damage to the facilities installed for the protection of the health and safety of workers employed in the establishment where he is employed.

A worker shall be required to use such means of protection and handle carefully whatever may be in his possession and to carry out the instructions laid down for the maintenance of his health and for his protection against the hazards of chemicals, gases and highly flammable liquids.

Article (20)

Any person who violates the provisions of this Decision shall be liable for the penalties provided for in Article (192) of the Labour Law in the Private Sector promulgated by Law No.(36) of 2012.

Article (21)

Decision No. (31) of 1977 regarding the Identification and Organisation of services and precautionary measures required to protect workers during work from the Hazards of Highly Flammable Liquids and Liquefied Gases shall be repealed..

Article (22)

The Undersecretary of the Ministry of labour shall implement this Decision, and it shall come into force from the day following the date of its publication in the Official Gazette.

The Minister of Labour

Jameel bin Mohamed Ali Humaidan

Issued on: 29 Rajab 1435 A.H

Corresponding to: 28 May 2014